# Arduino SI4635 DSP RADIO

Component Top Overlay

1-2-3

Jumper  -

1-2 : MW/LW/SW

2-3: FM Default

AM -FM

-

+

Front panel

Rotary Encoder

Oled 0.96”

Schematic Diagram

Components list

| Ref Name | Qty | Component |
| --- | --- | --- |
| Q3 | 1 | 2N3904 |
| Q4 | 1 | 2N3904 |
| PL2 | 1 | 2WP |
| PL6 | 1 | 2WP |
| LL | 1 | 2WP |
| PL5 | 1 | 3WP |
| CT2 | 1 | 3.5mm Stereo |
| PL3 | 1 | 4WP |
| PL4 | 1 | 4WP |
| PL1 | 1 | 5WP |
| U3 | 1 | 8DIL |
| U1 | 1 | 20SIL |
| U2 | 1 | 20SIL |
| IC3 | 1 | ATMEGA328P-PU |
| D2 | 1 | BAV199,215 |
| D3 | 1 | BAV199,215 |
| 1 | 1 | BNCPTH |
| U4 | 1 | BSS123 |
| U5 | 1 | BSS123 |
| U6 | 1 | BSS123 |
| C1 | 1 | C0805 – 22PF |
| C2 | 1 | C0805 – 22PF |
| C3 | 1 | C0805 – 22PF |
| C4 | 1 | C0805 – 22PF |
| C5 | 1 | C0805 – 1NF |
| C6 | 1 | C0805 – 470NF |
| C7 | 1 | C0805 – 22NF |
| C8 | 1 | C0805 – 100NF |
| C9 | 1 | C0805 – 100NF |
| C10 | 1 | C0805 – 100NF |
| C11 | 1 | C0805 – 100NF |
| C12 | 1 | C0805 – 100NF |
| C13 | 1 | C0805 – 100NF |
| C14 | 1 | C0805 – 100NF |
| C15 | 1 | C0805 – 100NF |
| C16 | 1 | C0805 – 100NF |
| C17 | 1 | C0805 – 100NF |
| C21 | 1 | C0805 – 100NF |
| C22 | 1 | C0805 – 100NF |
| C23 | 1 | C0805 – 1NF |
| C26 | 1 | C0805 – 10NF * |
| C18 | 1 | CP 1000UF 16V |
| C25 | 1 | CP 100UF 16V |
| C19 | 1 | CP1 4.7UF |
| C20 | 1 | CP1 4.7UF |
| C24 | 1 | CP1 10UF |
| C27 | 1 | CP1 10UF |
| C28 | 1 | CP1 10UF |
| C29 | 1 | CP1 10UF |
| Q1 | 1 | CRYSTALHC49UV 16MHZ |
| Q2 | 1 | CRYSTALTC26H 32.768 Khz |
| D1 | 1 | Diode 1N4001-7 |
| JP2 | 1 | JUMPER-2SMD-NC |
| JP3 | 1 | JUMPER-2SMD-NC |
| JP1 | 1 | JUMPER-3-OLD2-3 |
| S2 | 1 | LC1258EENP |
| D13 | 1 | LED |
| D4 | 1 | LL4148 – do not install if bav919 installed |
| D5 | 1 | LL4148– do not install if bav919 installed |
| D6 | 1 | LL4148– do not install if bav919 installed |
| D7 | 1 | LL4148– do not install if bav919 installed |
| VR1 | 1 | PEC11R-4220F-S0024 – rotary encoder |
| J4 | 1 | POWER_JACKPTH_LOCK |
| R1 | 1 | RESISTOR0805-RES 10K |
| R2 | 1 | RESISTOR0805-RES 5K6 *do not install |
| R3 | 1 | RESISTOR0805-RES 5K6 *do not install |
| R4 | 1 | RESISTOR0805-RES 10K * |
| R5 | 1 | RESISTOR0805-RES 10K * |
| R6 | 1 | RESISTOR0805-RES 10K * |
| R7 | 1 | RESISTOR0805-RES 10K * |
| R8 | 1 | RESISTOR0805-RES 4K7 |
| R9 | 1 | RESISTOR0805-RES 4K7 |
| R10 | 1 | RESISTOR0805-RES 4K7 |
| R11 | 1 | RESISTOR0805-RES 4K7 |
| R12 | 1 | RESISTOR0805-RES 10K |
| R13 | 1 | RESISTOR0805-RES 10K |
| R14 | 1 | RESISTOR0805-RES 47K |
| R15 | 1 | RESISTOR0805-RES 10K |
| R16 | 1 | RESISTOR0805-RES 10K |
| R17 | 1 | RESISTOR0805-RES 10R * |
| R18 | 1 | RESISTOR0805-RES 1K |
| R19 | 1 | RESISTOR0805-RES 100K |
| R20 | 1 | RESISTOR0805-RES 10K |
| R27 | 1 | RESISTOR0805-RES 1K1 |
| R21 | 1 | RESISTOR0805-RES 2.2K |
| SW1 | 1 | SW_FSM2JH tact switch |
| SW2 | 1 | SW_FSM2JH tact switch |
| SW3 | 1 | SW_FSM2JH tact switch |
| SW4 | 1 | SW_FSM2JH tact switch |
| SW6 | 1 | SW_FSM2JH tact switch |
| SW8 | 1 | SW_FSM2JH tact switch |
| SW5 | 1 | SW_FSM2JH tact switch |
| SW7 | 1 | SW_FSM2JH tact switch |
| SW9 | 1 | SW_FSM2JH tact switch |
| 4732/5 | 1 | SI4735-D60-GU OR GUR |
| IC1 | 1 | V_REG_LM1117SOT223 5V |
| IC2 | 1 | V_REG_LM1117SOT223 3V3 |

Note *: Omit do not install.

Connectors:

Battery:  Wire the 2-pin plug, the pins are shown in the table, rear most pin is positive.

+ Positive

-Negative

Speaker: Wire a 1-Watt speaker 8 ohm to the 2 pin Plug. The pins are shown in the table. rear most pin is negative ground.

-Negative

+ Positive

Assembling the board Instructions

Front Panel

- Start by Installing the 9 switches on the side where the function is printed – bottom side.

- Check that all switches are installed correctly pressed down and then proceed with the installation the OLED 0.96” display.

- Handle with care the OLED display as it is very fragile and install the rotary encoder. Insert the 4 pin connector and solder on the pcb then level the OLED and solder the 4 terminals

- Solder the two capacitors c22, c23 on the back side of the PCB.

- Install the 20-pin connector and solder it to main board.  The connector must be connected from the bottom side of the mainboard.

Main Board

- Start by installing the surface mount capacitors 0805

- Install the capacitors C1,C2,C3,C4 - 22pf

- Install the capacitors decoupling C8,C9,C10,C11,C12,C13,C14,C15,C16,C17,C21,C22 - 100nf –

- Install the capacitors C5,C23 – 1nF

- Install the capacitors C6 – 470nF

- Install the capacitors C7– 22nF 0603

- Install the resistors R8,R9,R10,R11 - 4K7

- Install the resistors R1,R15,R16,R19 - 10K

- Install the resistors R20 - 100K

- Install the resistors R12 - 1K

- Install the voltage regulators 5v and 3.3v  LM1117 , Take care to install IC1 with the 5v, and IC2 3.3v.

- Install the si4735 chip. Observe pin 1 ! solder it with a hot air soldering iron and apply flux

- Install the two crystals Q1-16Mhz , Q2 – 32.768Khz .

- On the radio crystal secure it with a wire loop - pads are on the top part and solder each side

- Install the smd mosfets U4,U5,U6- for version 1.0A the daughter board is already assembled.

- For version 1.0A install 3.3v Zener in place of R13, Observe the polarity of the diode cathode to rear of board

- Install the IC sockets 28 pin and 8 pin IC3, U3

- Install the Q3, Q4 transistors, observe their pinout – emitter to ground.

- Install the through hole capacitors C24,C27,C28,C29 – 10uF.

- Install the through hole capacitors C19,C20 – 4.7uF.

- Install the through hole capacitors C18 – 1000uF.

- Install the through hole capacitors C25 – 100uF.

- Install the protection diodes D2,D3 or else the individual diodes D4 to D7

- For version 1.0A - Install the i2c level shifter See page 7

- Install the remaining components, switch, connectors, plugs.

Once everything is installed – connect an 8.4v power source and measure the voltage on the tab on ic1 and ic2 – you should get 5v and 3v3, respectively.

Install the 28 pin chip, the firmware is already loaded, observe pin 1 facing c11

Power on and the display will show the splash screen and defaults to a radio mode.

Please visit Ricardo Page , where you will find a mine of resources.

Level Shifter installation Guide.

For version 1.0A

Although the i2c bus communication between the 5-volt Arduino and the 3v3 dsp chip was tested with a resistor in series 1k on the bus – it was noted that sometimes lockup of the chip was caused by this type of interfacing.

To have a reliable communication a level shifter is installed so as it will translate the 5v signal into a 3v3 signal making the interfacing of both devices more stable and safer.

5 Volts

3.3Volts

The level shifter board interfaces to the SCL and SDA  lines.

3.3V_SCL

3.3_SDA

To A5 Arduino

To A4 Arduino

Arduino pin A4,A5  connect to the 5volt header  and DSP chip pins SCL,SDA  connect to the 3v3 header

Step 1 – prepare the board – apply solder to the four pins as per picture.

Step 2 – prepare/cut three wires with length of 4.2cms and 1 wire  bare of 2.5cms

Step 3- prepare two bare wires of 1.5cms as per picture.

Step 4- solder the  pieces of wire to the pcb as per picture

Step 5- place the level shifter board vertically with the 3v3 header facing the mainboard pcb as per picture.

Solder the three wires to the level shifter board.

Note : R4 and R6 must be wired as per table

| R6 PIN | R6 PIN |  | R4 PIN | R4 PIN |
| --- | --- | --- | --- | --- |
| 3V3_I2c SCL pin 2 bottom side | 5V I2C SCL pin 2 top side |  | 3V3_I2c SDA pin 3 bottom side | 5V I2C SDA pin 3 top side |

Step 6 – Insert the three wires into the 5 Volt header as per picture.  First pin is the 5volts supply the two remaining wires follow the path from the bottom header pin 2 in – pin 2 out -  pin 3 in – pin 3 out.

Cut the excess wire and secure with some hot glue on the back. Apply some hot glue and install the printed cover onto the level shifter.

Button Mapping

| Show Battery State | Soft Mute in AM/LW/SW | BFO/VFO |
| --- | --- | --- |
| Volume control | Band control AM/LW/SW/FM | AGC |
| SSB MODE LSB/USB | Bandwidth | Step |